#include <AccelStepper.h>

// Pin definitions because setting them manually is less stressful right now

#define DIR_PIN_1 11

#define STEP_PIN_1 12

#define SWITCH_PIN_1 38

#define LED_PIN_1 3

#define DIR_PIN_2 13

#define STEP_PIN_2 14

#define SWITCH_PIN_2 36

#define LED_PIN_2 4

#define DIR_PIN_3 15

#define STEP_PIN_3 16

#define SWITCH_PIN_3 32

#define LED_PIN_3 5

#define DIR_PIN_4 17

#define STEP_PIN_4 18

#define SWITCH_PIN_4 30

#define LED_PIN_4 6

#define MOTOR_INTERFACE_TYPE 1

// Motor states

enum MotorState {

IDLE,

MOVING_TO_HOME,

BACKING_OFF_SWITCH,

REVERSING,

MOVING_TO_POSITION,

STOPPED

};

class StepperMotor {

private:

AccelStepper stepper;

int homeSwitchPin;

int ledPin;

MotorState state;

bool previousSwitchState;

public:

StepperMotor(int interface, int step, int dir, int homeSwitch, int led)

: stepper(interface, step, dir),

homeSwitchPin(homeSwitch),

ledPin(led),

state(IDLE),

previousSwitchState(true) {

pinMode(homeSwitchPin, INPUT_PULLUP);

pinMode(ledPin, OUTPUT);

stepper.setMaxSpeed(2000.0); // Adjust the maximum speed (steps per second)

stepper.setAcceleration(1000.0); // Adjust the acceleration (steps per second per second)

stepper.setSpeed(1000.0); // Set the initial speed

}

void moveToHome() {

if (state == IDLE) {

state = MOVING_TO_HOME;

stepper.moveTo(40000); // Move a fixed number of steps in the desired direction

}

}

void moveToPosition() {

if (state == STOPPED) {

state = MOVING_TO_POSITION;

stepper.moveTo(-14000); // Move to the specific position

}

}

void update() {

bool currentSwitchState = digitalRead(homeSwitchPin);

if (state == MOVING_TO_HOME) {

digitalWrite(ledPin, HIGH);

if (currentSwitchState != previousSwitchState) {

// Home switch triggered

state = BACKING_OFF_SWITCH;

stepper.stop(); // Stop the motor

stepper.setCurrentPosition(0); // Reset the current position

}

} else if (state == BACKING_OFF_SWITCH) {

digitalWrite(ledPin, LOW);

if (currentSwitchState == previousSwitchState) {

// Switch no longer triggered

state = REVERSING;

stepper.setSpeed(-500); // Set a constant speed in the opposite direction (adjust as needed)

stepper.moveTo(-200); // Move a fixed number of steps in the opposite direction (adjust as needed)

}

} else if (state == REVERSING) {

digitalWrite(ledPin, LOW);

if (!stepper.isRunning()) {

// Movement in the opposite direction completed

state = STOPPED;

stepper.setCurrentPosition(0); // Reset the current position

}

} else if (state == STOPPED) {

digitalWrite(ledPin, HIGH);

moveToPosition(); // We move to the position after we are stopped

} else if (state == MOVING_TO_POSITION) {

digitalWrite(ledPin, HIGH);

if (!stepper.isRunning()) {

// Position reached

state = MOVING_TO_HOME;

stepper.setCurrentPosition(0); // Reset the current position

stepper.setSpeed(1000); // Set the speed for moving to home

stepper.moveTo(40000); // Move all the way back to home

}

}

// Debug output

Serial.print("Home switch state: ");

Serial.println(currentSwitchState == HIGH ? "HIGH" : "LOW");

Serial.print("Motor state: ");

switch (state) {

case IDLE: Serial.println("IDLE"); break;

case MOVING_TO_HOME: Serial.println("MOVING_TO_HOME"); break;

case BACKING_OFF_SWITCH: Serial.println("BACKING_OFF_SWITCH"); break;

case REVERSING: Serial.println("REVERSING"); break;

case MOVING_TO_POSITION: Serial.println("MOVING_TO_POSITION"); break; // Add this case

case STOPPED: Serial.println("STOPPED"); break;

}

stepper.run();

previousSwitchState = currentSwitchState;

}

void debugPrint() {

bool currentSwitchState = digitalRead(homeSwitchPin);

// Debug output

Serial.print("Home switch state: ");

Serial.println(currentSwitchState == HIGH ? "HIGH" : "LOW");

Serial.print("Motor state: ");

switch (state) {

case IDLE: Serial.println("IDLE"); break;

case MOVING_TO_HOME: Serial.println("MOVING_TO_HOME"); break;

case BACKING_OFF_SWITCH: Serial.println("BACKING_OFF_SWITCH"); break;

case REVERSING: Serial.println("REVERSING"); break;

case MOVING_TO_POSITION: Serial.println("MOVING_TO_POSITION"); break; // Add this case

case STOPPED: Serial.println("STOPPED"); break;

}

}

};

StepperMotor motor1(MOTOR_INTERFACE_TYPE, STEP_PIN_1, DIR_PIN_1, SWITCH_PIN_1, LED_PIN_1);

StepperMotor motor2(MOTOR_INTERFACE_TYPE, STEP_PIN_2, DIR_PIN_2, SWITCH_PIN_2, LED_PIN_2);

StepperMotor motor3(MOTOR_INTERFACE_TYPE, STEP_PIN_3, DIR_PIN_3, SWITCH_PIN_3, LED_PIN_3);

StepperMotor motor4(MOTOR_INTERFACE_TYPE, STEP_PIN_4, DIR_PIN_4, SWITCH_PIN_4, LED_PIN_4);

void setup() {

delay(250);

Serial.begin(9600);

motor1.moveToHome();

motor2.moveToHome();

motor3.moveToHome();

motor4.moveToHome();

}

void loop() {

motor1.update();

motor2.update();

motor3.update();

motor4.update();

motor1.debugPrint();

}