//Roller Racer - Motor Controller Brain

//Target Winter Street Studio

//June 2023

//B.Severns

//libraries

#include <AccelStepper.h>

#include <Adafruit_NeoPixel.h>

#include <Audio.h>

#include <SD.h>

#include <SPI.h>

#include <Arduino.h>

#include <elapsedMillis.h>

//constants

#define LED_PIN 23

#define LED_COUNT 42

#define N_STEPPERS 4

#define HOME_SENSOR(i) ((i < 2) ? (30 + (i * 2)) : (36 + (i - 2) * 2))

#define STEP_PIN(i) ((i)*2 + 12)

#define DIR_PIN(i) ((i)*2 + 11)

#define MAX_POSITION 14000

#define BACKOFF_STEPS 150

#define LANE_COUNT 4

// Audio library objects

AudioPlaySdWav playWav1; // for the startGame sound

AudioPlaySdWav playWav2; // for the rollerGame sound

AudioPlaySdWav playWav3; // for the ballHit sound

AudioOutputI2SOct audioOutput;

AudioMixer4 mixer1; // Add a mixer

AudioMixer4 mixer2; // Add a second mixer

AudioMixer4 mixer3; // Add a third mixer

AudioConnection patchCord1(playWav1, 0, mixer1, 0); // Connect playWav1 to the first mixer

AudioConnection patchCord2(playWav1, 1, mixer1, 1); // Connect playWav1 to the first mixer

AudioConnection patchCord3(playWav2, 0, mixer2, 0); // Connect playWav2 to the second mixer

AudioConnection patchCord4(playWav2, 1, mixer2, 1); // Connect playWav2 to the second mixer

AudioConnection patchCord5(playWav3, 0, mixer3, 0); // Connect playWav3 to the third mixer

AudioConnection patchCord6(playWav3, 1, mixer3, 1); // Connect playWav3 to the third mixer

AudioConnection patchCord7(mixer1, audioOutput); // Connect the first mixer to the output

AudioConnection patchCord8(mixer2, audioOutput); // Connect the second mixer to the output

AudioConnection patchCord9(mixer3, audioOutput); // Connect the third mixer to the output

Adafruit_NeoPixel strip(LED_COUNT, LED_PIN, NEO_GRB + NEO_KHZ800); //neopixel strips: 21-21

AccelStepper steppers[N_STEPPERS] = {

AccelStepper(1, STEP_PIN(3), DIR_PIN(3)),

AccelStepper(1, STEP_PIN(2), DIR_PIN(2)),

AccelStepper(1, STEP_PIN(1), DIR_PIN(1)),

AccelStepper(1, STEP_PIN(0), DIR_PIN(0))

};

const int homeButtonPin = 2;

bool isHomed[N_STEPPERS] = { false };

bool reverseDirection[N_STEPPERS] = { false };

bool isGameStarted = false;

bool resettingGame = false;

bool sendingData = false;

bool laneState = false;

uint8_t winningLane = 0;

volatile bool buttonPressed = false; // This variable will be changed in the ISR

// Variables to hold the previous state of the home sensors

volatile bool homeSensorState[N_STEPPERS] = { false };

bool buttonState = HIGH; // current state of the button

bool lastButtonState = HIGH; // previous state of the button

unsigned long buttonPressTime = 0; // time of the last button press

unsigned long debounceDelay = 50; // debounce delay in milliseconds

const int LANE_IND[LANE_COUNT] = { 3, 4, 5, 6 };

elapsedMillis winningLaneBlinkTimer;

const unsigned long winningLaneBlinkInterval = 500;

enum State {

Initialising,

WaitingToStart,

Playing,

GameOver,

Resetting

};

State currentState = Initialising;

enum LaneColor {

LANE_TEAL,

LANE_ORANGE_YELLOW,

LANE_PURPLE,

LANE_GREEN

};

bool allSteppersHomed() {

for (int i = 0; i < N_STEPPERS; i++) {

if (!isHomed[i]) {

return false;

}

}

return true;

}

bool getLaneIndicator(int laneId) {

if (laneId >= 0 && laneId < LANE_COUNT) {

return digitalRead(LANE_IND[laneId]) == HIGH;

}

return false;

}

bool isStepperMoving(int i) {

if (i >= 0 && i < N_STEPPERS) {

return steppers[i].isRunning();

}

return false;

}

bool anyStepperMoving() {

for (int i = 0; i < N_STEPPERS; i++) {

if (steppers[i].isRunning()) {

return true;

}

}

return false;

}

const uint32_t laneColors[] = {

strip.Color(0, 210, 255), // Teal - lane 1

strip.Color(255, 210, 0), // Orange-Yellow - lane 2

strip.Color(210, 0, 255), // Purple - lane 3

strip.Color(170, 210, 0) // Green - lane 4

};

bool startGame() {

// Check if all steppers are homed

if (!allSteppersHomed()) {

Serial.println("Cannot start game. All steppers must be homed first.");

return false;

}

// Send 'H' command to sensor hubs to turn their light on

Serial1.write('H'); // Assuming lane 1 is connected to Serial1

Serial6.write('H'); // Assuming lane 2 is connected to Serial6

Serial7.write('H'); // Assuming lane 3 is connected to Serial6

Serial8.write('H'); // Assuming lane 4 is connected to Serial7

Serial.println("Game started");

playWav1.play("startGame.wav");

delay(2200);

playWav2.play("rollerGame.wav");

startChase(strip.Color(255, 0, 0)); // Red color chase

return true;

}

void startChase(uint32_t color) {

while (isGameStarted) {

for (int i = 0; i < LED_COUNT / 2; i++) {

strip.setPixelColor(i, color);

strip.setPixelColor(LED_COUNT - 1 - i, color);

strip.show();

delay(100);

strip.setPixelColor(i, 0);

strip.setPixelColor(LED_COUNT - 1 - i, 0);

strip.show();

}

}

}

void setLaneIndicator(int laneId, bool isOk) {

if (laneId >= 0 && laneId < LANE_COUNT) {

digitalWrite(LANE_IND[laneId], isOk ? HIGH : LOW);

}

}

void endGame(int laneId) {

strip.clear();

// Set the winning lane color

if (laneId >= 0 && laneId < N_STEPPERS) {

winningLane = laneId;

strip.fill(laneColors[winningLane], 0, LED_COUNT);

}

strip.show();

Serial.println("Game ended");

homeAllSteppers();

resettingGame = true; // Set flag to start the resetting process

// Send code to the corresponding sensor hub to toggle the light every half second

switch (laneId) {

case 0:

Serial1.write('T'); // Assuming lane 1 is connected to Serial1

Serial6.write('O'); // Turn off lane 2 light

Serial7.write('O'); // Turn off lane 3 light

Serial8.write('O'); // Turn off lane 4 light

break;

case 1:

Serial1.write('O'); // Turn off lane 1 light

Serial6.write('T'); // Assuming lane 2 is connected to Serial6

Serial7.write('O'); // Turn off lane 3 light

Serial8.write('O'); // Turn off lane 4 light

break;

case 2:

Serial1.write('O'); // Turn off lane 1 light

Serial6.write('O'); // Turn off lane 2 light

Serial7.write('T'); // Assuming lane 3 is connected to Serial6

Serial8.write('O'); // Turn off lane 4 light

break;

case 3:

Serial1.write('O'); // Turn off lane 1 light

Serial6.write('O'); // Turn off lane 2 light

Serial7.write('O'); // Turn off lane 3 light

Serial8.write('T'); // Assuming lane 4 is connected to Serial7

break;

default: break;

}

delay(500); // Delay for 0.5 seconds before toggling the light

setLaneIndicator(laneId, !getLaneIndicator(laneId)); // Toggle the light on the winning lane

// Reset the timer

winningLaneBlinkTimer = 0;

}

// Function to handle the home sensor interrupt

void handleHomeSensorInterrupt(int i) {

// Read the current state of the home sensor

bool currentState = digitalRead(HOME_SENSOR(i));

// If the state has changed from the previous state

if (currentState != homeSensorState[i]) {

homeSensorState[i] = currentState;

// If the home sensor is triggered (LOW state)

if (currentState == LOW) {

steppers[i].stop();

steppers[i].setSpeed(0);

steppers[i].setCurrentPosition(0);

reverseDirection[i] = true;

isHomed[i] = true;

Serial.println("Stepper " + String(i) + " reached home position");

}

}

}

void setup() {

strip.begin(); //start the neopixel

strip.show();

for (int i = 0; i < N_STEPPERS; i++) { //setup the motors/home sensors

pinMode(HOME_SENSOR(i), INPUT_PULLUP);

steppers[i].setMaxSpeed(800.0);

steppers[i].setAcceleration(500.0);

steppers[i].move(-MAX_POSITION);

}

Serial.begin(9600); // Serial monitor

Serial.println("Setup initialized");

Serial1.begin(9600); // Serial1 for lane 1

Serial6.begin(9600); // Serial6 for lane 2

Serial7.begin(9600); // Serial7 for lane 3

Serial8.begin(9600); // Serial8 for lane 4

AudioMemory(12);

pinMode(homeButtonPin, INPUT_PULLUP);

// Attach an interrupt to digital pin 2, on falling edge (from HIGH to LOW)

attachInterrupt(digitalPinToInterrupt(homeButtonPin), buttonPress, RISING);

for (int i = 0; i < LANE_COUNT; ++i) {

pinMode(LANE_IND[i], OUTPUT);

digitalWrite(LANE_IND[i], LOW);

delay(5);

digitalWrite(LANE_IND[i], HIGH);

delay(50);

digitalWrite(LANE_IND[i], LOW);

}

// Attach interrupts for each home sensor

attachInterrupt(digitalPinToInterrupt(HOME_SENSOR(0)), handleHomeSensorInterrupt0, CHANGE);

attachInterrupt(digitalPinToInterrupt(HOME_SENSOR(1)), handleHomeSensorInterrupt1, CHANGE);

attachInterrupt(digitalPinToInterrupt(HOME_SENSOR(2)), handleHomeSensorInterrupt2, CHANGE);

attachInterrupt(digitalPinToInterrupt(HOME_SENSOR(3)), handleHomeSensorInterrupt3, CHANGE);

currentState = WaitingToStart; // Set the initial state

// Print the initial state

switch (currentState) {

case Initialising:

Serial.println("Initialising state");

homeAllSteppers();

break;

case WaitingToStart:

Serial.println("WaitingToStart state");

break;

case Playing:

Serial.println("Playing state");

break;

case GameOver:

Serial.println("GameOver state");

break;

case Resetting:

Serial.println("Resetting state");

break;

}

}

void loop() {

// Read the button state

int reading = digitalRead(homeButtonPin);

// Check for a button press with debouncing

if (reading != lastButtonState) {

buttonPressTime = millis(); // record the time of the button press

}

// Check if the game has started

if (!isGameStarted && reading == LOW && millis() - buttonPressTime > debounceDelay) {

if (startGame()) {

currentState = Playing; // Transition to the Playing state

Serial.println("Button pressed. Transitioning to Playing state.");

} else {

Serial.println("Button pressed, but game could not be started.");

}

}

lastButtonState = reading;

switch (currentState) {

case WaitingToStart:

digitalWrite(LANE_IND[0], HIGH);

digitalWrite(LANE_IND[1], LOW);

digitalWrite(LANE_IND[2], LOW);

digitalWrite(LANE_IND[3], LOW);

// Check if the button is pressed to transition to the Playing state

if (isGameStarted && buttonPressed) {

buttonPressed = false; // Reset the flag

currentState = Playing; // Transition to the Playing state

Serial.println("Button pressed. Transitioning to Playing state.");

}

break;

case Playing:

// Check if any stepper has reached the maximum position

isGameStarted = true; // Set isGameStarted to true when in the Playing state

digitalWrite(LANE_IND[1], HIGH);

digitalWrite(LANE_IND[0], LOW);

digitalWrite(LANE_IND[2], LOW);

digitalWrite(LANE_IND[3], LOW);

for (int i = 0; i < N_STEPPERS; i++) {

if (abs(steppers[i].currentPosition()) >= MAX_POSITION) {

endGame(i);

currentState = GameOver;

Serial.println("End of game, stepper reached maximum position");

break;

}

}

break;

case GameOver:

digitalWrite(LANE_IND[3], HIGH);

digitalWrite(LANE_IND[1], LOW);

digitalWrite(LANE_IND[0], LOW);

digitalWrite(LANE_IND[2], LOW);

if (winningLaneBlinkTimer > winningLaneBlinkInterval) {

// Toggle the winning lane LED every half second

setLaneIndicator(winningLane, !getLaneIndicator(winningLane));

Serial.println("Game over");

// Reset the timer

winningLaneBlinkTimer = 0;

}

break;

case Resetting:

digitalWrite(LANE_IND[2], HIGH);

digitalWrite(LANE_IND[0], LOW);

digitalWrite(LANE_IND[1], LOW);

digitalWrite(LANE_IND[3], LOW);

homeAllSteppers();

if (!anyStepperMoving()) {

isGameStarted = false;

resettingGame = false;

currentState = WaitingToStart;

}

break;

}

// Check if the homeButton is pressed to transition from WaitingToStart to Playing

if (currentState == WaitingToStart && buttonPressed && allSteppersHomed()) {

currentState = Playing; // Transition to the Playing state

buttonPressed = false; // Reset the buttonPressed flag

Serial.println("Button pressed. Transitioning to Playing state.");

}

if (isGameStarted) {

for (int i = 0; i < N_STEPPERS; i++) {

steppers[i].run();

}

}

if (resettingGame && !anyStepperMoving()) {

isGameStarted = false;

resettingGame = false;

}

if (currentState == Resetting) {

for (int i = 0; i < N_STEPPERS; i++) {

handleHoming(i); // Call the new handleHoming() function

}

}

delay(10);

}

void handleHoming(int i) {

if (!isHomed[i]) {

if (digitalRead(HOME_SENSOR(i)) == LOW) {

steppers[i].stop();

steppers[i].setSpeed(0);

steppers[i].setCurrentPosition(0);

reverseDirection[i] = true;

isHomed[i] = true;

Serial.println("Stepper " + String(i) + " reached home position");

} else {

steppers[i].setSpeed(-100);

steppers[i].setAcceleration(500);

steppers[i].runSpeed();

Serial.println("Stepper " + String(i) + " is homing...");

}

} else {

if (digitalRead(HOME_SENSOR(i)) == LOW && !reverseDirection[i]) {

steppers[i].move(MAX_POSITION);

reverseDirection[i] = true;

Serial.println("Stepper " + String(i) + " is moving towards the maximum position...");

} else if (digitalRead(HOME_SENSOR(i)) == HIGH && reverseDirection[i]) {

steppers[i].stop();

steppers[i].setCurrentPosition(MAX_POSITION);

reverseDirection[i] = false;

isHomed[i] = false;

Serial.println("Stepper " + String(i) + " returned to the starting position");

} else {

steppers[i].setSpeed(100);

steppers[i].setAcceleration(500);

steppers[i].runSpeed();

Serial.println("Stepper " + String(i) + " is moving...");

}

}

}

// Function to handle the home sensor interrupt

void handleHomeSensorInterrupt0() {

handleHomeSensorInterrupt(0);

}

void handleHomeSensorInterrupt1() {

handleHomeSensorInterrupt(1);

}

void handleHomeSensorInterrupt2() {

handleHomeSensorInterrupt(2);

}

void handleHomeSensorInterrupt3() {

handleHomeSensorInterrupt(3);

}

void homeAllSteppers() {

Serial.println("Going home!");

for (int i = 0; i < N_STEPPERS; i++) {

isHomed[i] = false;

reverseDirection[i] = false;

steppers[i].move(-MAX_POSITION); // start all motors moving towards home

}

}

// This is the ISR that gets called when the button is pressed

void buttonPress() {

buttonPressed = true;

}