#include <AccelStepper.h>

#define DIR_PIN 11

#define STEP_PIN 12

#define SWITCH_PIN 38

#define LED_IDLE_PIN 3

#define LED_MOVING_TO_HOME_PIN 4

#define LED_BACKING_OFF_SWITCH_PIN 5

#define MOTOR_INTERFACE_TYPE 1

enum MotorState {

IDLE,

MOVING_TO_HOME,

BACKING_OFF_SWITCH,

REVERSING,

STOPPED

};

class StepperMotor {

private:

AccelStepper stepper;

int homeSwitchPin;

int ledIdlePin;

int ledMovingToHomePin;

int ledBackingOffSwitchPin;

MotorState state;

bool previousSwitchState;

bool homed;

public:

StepperMotor(int interface, int step, int dir, int homeSwitch, int ledIdle, int ledMovingToHome, int ledBackingOffSwitch)

: stepper(interface, step, dir),

homeSwitchPin(homeSwitch),

ledIdlePin(ledIdle),

ledMovingToHomePin(ledMovingToHome),

ledBackingOffSwitchPin(ledBackingOffSwitch),

state(IDLE),

previousSwitchState(true),

homed(false) {

pinMode(homeSwitchPin, INPUT_PULLUP);

pinMode(ledIdlePin, OUTPUT);

pinMode(ledMovingToHomePin, OUTPUT);

pinMode(ledBackingOffSwitchPin, OUTPUT);

stepper.setMaxSpeed(2000.0);

stepper.setAcceleration(1000.0);

stepper.setSpeed(1000.0);

}

void moveToHome() {

if (!homed) {

state = MOVING_TO_HOME;

digitalWrite(ledIdlePin, LOW);

digitalWrite(ledMovingToHomePin, HIGH);

digitalWrite(ledBackingOffSwitchPin, LOW);

stepper.moveTo(40000);

}

}

void update() {

bool currentSwitchState = digitalRead(homeSwitchPin);

if (state == MOVING_TO_HOME) {

if (!previousSwitchState && currentSwitchState) {

state = BACKING_OFF_SWITCH;

stepper.stop();

stepper.setCurrentPosition(0);

}

} else if (state == BACKING_OFF_SWITCH) {

if (previousSwitchState && !currentSwitchState) {

state = REVERSING;

stepper.setSpeed(-500);

stepper.moveTo(-200);

}

} else if (state == REVERSING) {

if (!stepper.isRunning()) {

state = STOPPED;

stepper.setCurrentPosition(0);

}

} else if (state == STOPPED || state == IDLE) {

if (!previousSwitchState && currentSwitchState) {

state = MOVING_TO_HOME;

stepper.setSpeed(1000);

stepper.moveTo(40000);

}

}

digitalWrite(ledIdlePin, state == STOPPED || state == IDLE);

digitalWrite(ledMovingToHomePin, state == MOVING_TO_HOME);

digitalWrite(ledBackingOffSwitchPin, state == BACKING_OFF_SWITCH || state == REVERSING);

previousSwitchState = currentSwitchState;

stepper.run();

if (state == STOPPED) {

homed = true;

}

}

bool getHasHomed() {

return homed; // Function to return the homed state

}

void move(int steps) { // Function to move a certain number of steps

if (state == IDLE) {

stepper.move(steps);

}

}

void setSpeed(int speed) { // Function to set the speed

if (state == IDLE) {

stepper.setSpeed(speed);

}

}

void stop() { // Function to stop the stepper

if (state != MOVING_TO_HOME && state != BACKING_OFF_SWITCH) {

stepper.stop();

}

}

};

StepperMotor motor(MOTOR_INTERFACE_TYPE, STEP_PIN, DIR_PIN, SWITCH_PIN, LED_IDLE_PIN, LED_MOVING_TO_HOME_PIN, LED_BACKING_OFF_SWITCH_PIN);

void setup() {

Serial.begin(9600);

motor.moveToHome();

}

void loop() {

motor.update();

// Add the serial controls back

if (Serial.available()) {

String command = Serial.readStringUntil('\n');

if (command.startsWith("move")) {

int steps = command.substring(5).toInt();

motor.move(steps);

} else if (command.startsWith("speed")) {

int speed = command.substring(6).toInt();

motor.setSpeed(speed);

} else if (command.startsWith("stop")) {

motor.stop();

}

}

}