#include <VL53L0X.h>

#define NUM_SENSORS 6

#define NUM_GROUPS 3

#define lightR 13 // turn the lane light on/off

const int xshutPins[NUM_SENSORS] = { 4, 5, 6, 7, 10, 11 };

const int groupStart[NUM_GROUPS] = { 0, 4, 5 };

const int groupEnd[NUM_GROUPS] = { 3, 4, 5 };

const int minRange = 75;

const int maxRange = 100;

const int time = 25;

VL53L0X sensors[NUM_SENSORS];

unsigned long lastTriggeredTime[NUM_GROUPS] = { 0, 0, 0 };

char lastCommand = 'N';

char laneAddress = '1'; //change this per installation

bool sensorInitSuccessful[NUM_SENSORS] = { false };

void setup() {

Serial.begin(9600); // For debugging

Serial1.begin(9600); // Serial1 for communication on pins 7 (RX) and 8 (TX)

pinMode(lightR, OUTPUT);

for (int i = 0; i < NUM_SENSORS; i++) {

pinMode(xshutPins[i], OUTPUT);

digitalWrite(xshutPins[i], LOW);

}

for (int i = 0; i < NUM_SENSORS; i++) {

digitalWrite(xshutPins[i], HIGH);

delay(10);

sensorInitSuccessful[i] = sensors[i].init(true);

bool sensorInitSuccessful = sensors[i].init(true); // Move this line inside the loop

if (!sensorInitSuccessful) { // This line should be here to check for each sensor

Serial.print("Sensor ");

Serial.print(i);

Serial.println(" failed to initialize!");

while (1)

; // Stay in an infinite loop

}

sensors[i].setAddress(0x30 + i);

sensors[i].startContinuous();

Serial.print("Sensor ");

Serial.print(i);

Serial.println(" initialized successfully with its respective I2C address!");

digitalWrite(lightR, HIGH);

delay(time - 10);

digitalWrite(lightR, LOW);

}

}

void loop() {

digitalWrite(lightR, LOW;

// Check if there is data available on Serial1

if (Serial1.available() > 0) {

char command = Serial1.read();

switch (command) {

case 'H': // Home command

digitalWrite(lightR, HIGH); // Turn on the light

break;

case 'T':

for (int d = 0; d < 35; d++) {

digitalWrite(lightR, !digitalRead(lightR));

delay(time * 20);

}

break;

case 'O':

digitalWrite(lightR, LOW);

break;

case 'D': // Send data command

// No need to include sensor reading code here

break;

case 'R': // Report command

// Check the status of the sensors and return a message

for (int i = 0; i < NUM_SENSORS; i++) {

if (sensorInitSuccessful[i]) {

Serial.println("SENSOR_OK");

} else {

Serial.println("SENSOR_ERROR");

}

}

default:

break;

}

}

// Read sensor data and print to the serial monitor

for (int g = 0; g < NUM_GROUPS; g++) {

for (int i = groupStart[g]; i <= groupEnd[g]; i++) {

uint16_t range = sensors[i].readRangeContinuousMillimeters();

if (!sensors[i].timeoutOccurred()) {

if (range > minRange && range < maxRange && millis() - lastTriggeredTime[g] > (time * 2)) {

digitalWrite(lightR, LOW);

if (g == 0) {

lastCommand = 'F';

} else if (g == 1) {

lastCommand = 'M';

} else if (g == 2) {

lastCommand = 'L';

}

lastTriggeredTime[g] = millis();

} else {

lastCommand = 'N';

}

Serial1.write(lastCommand); // Send the command to Serial1

unsigned long startTime = millis();

bool acknowledgmentReceived = false;

// Wait for acknowledgment or timeout

while (millis() - startTime < (time * 4)) { //change the number here to fine-tune the machine

if (Serial1.available() > 0) {

char ack = Serial1.read();

if (ack == 'A') {

acknowledgmentReceived = true;

break;

}

if (ack == 'R') { // Report command

// Check the status of the sensors and return a message

if (sensorInitSuccessful) {

Serial.println("SENSOR_OK");

} else {

Serial.println("SENSOR_ERROR");

}

}

}

}

if (acknowledgmentReceived) {

// 'A' acknowledgment received, proceed with next sensor

continue;

}

} else { // This should be in the for loop

Serial.print("Sensor ");

Serial.print(i);

Serial.println(" reading timed out!");

continue;

}

// Print sensor data to the serial monitor

Serial.print("Sensor ");

Serial.print(i);

Serial.print(": ");

Serial.print(range);

Serial.println(" mm");

}

}

delay(time * 2);

}